# Lab 09 – Worksheet

| Name:Rushan Ali, Fardeen | ID: | Section: |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Truth Table
a) Main Control Unit

b) ALU Control Unit

Instruction / Opcode RW AS MR MW M2R Br ALUOp R-Type (0110011) 1 0 0 0 0 0 10 ADDI (0010011) 1 1 0 0 0 0 00 LW/LH/LB (0000011) 1 1 1 0 1 0 00 SW/SH/SB (0100011) 0 1 0 1 X 0 00 BEQ (1100011) 0 0 0 0 X 1 01 ALUOp funct3 f7[5] Operation ALUCtrl 00 XXX X Add (L/S/ADDI) 0010 01 XXX X Sub (BEQ) 0110 10 000 0 ADD (R-type) 0010 10 000 1 SUB (R-type) 0110 10 001 X SLL 0100 10 101 X SRL 0101 10 111 X AND 0000 10 110 X OR 0001 10 100 X XOR 0011

Comments

## Task 2

Design Module Codes for Main Control Unit, and ALU Control Unit.

module TopModule ( input [15:0] sw, output [15:0] led ); wire [6:0] opcode; wire [2:0] funct3; wire funct7_5; wire RegWrite; wire ALUSrc; wire MemRead; wire MemWrite; wire MemtoReg; wire Branch; wire [1:0] ALUOp; wire [3:0] ALUControl_out; // Connect switches to instruction fields assign opcode = sw[6:0]; assign funct3 = sw[9:7]; assign funct7_5 = sw[10]; // Instantiate Main Control Unit (matching maincontrol.v) maincontrol main_ctrl ( .opcode(opcode), .RegWrite(RegWrite), .ALUSrc(ALUSrc), .MemRead(MemRead), .MemWrite(MemWrite), .MemtoReg(MemtoReg), .Branch(Branch), .ALUOp(ALUOp) ); // Instantiate ALU Control Unit (matching Alucontrol.v) Alucontrol alu_ctrl ( .ALUOp(ALUOp), .funct3(funct3), .funct7_5(funct7_5), .ALUControl_out(ALUControl_out) ); // Map control outputs to onboard LEDs assign led[0] = RegWrite; assign led[1] = ALUSrc; assign led[2] = MemRead; assign led[3] = MemWrite; assign led[4] = MemtoReg; assign led[5] = Branch; assign led[9:6] = ALUControl_out; assign led[11:10] = ALUOp; assign led[15:12] = 4'b0000; // Unused LEDs endmodule module Alucontrol ( input [1:0] ALUOp, input [2:0] funct3, input funct7_5, // Bit 5 of funct7 output reg [3:0] ALUControl_out ); always @(*) begin case (ALUOp) 2'b00: ALUControl_out = 4'b0010; // ADD (for loads/stores & ADDI) 2'b01: ALUControl_out = 4'b0110; // SUB (for BEQ) 2'b10: begin // R-type operations case (funct3) 3'b000: begin if (funct7_5 == 1'b1) ALUControl_out = 4'b0110; // SUB else ALUControl_out = 4'b0010; // ADD end 3'b001: ALUControl_out = 4'b0100; // SLL[cite: 1] 3'b101: ALUControl_out = 4'b0101; // SRL[cite: 1] 3'b111: ALUControl_out = 4'b0000; // AND[cite: 1] 3'b110: ALUControl_out = 4'b0001; // OR[cite: 1] 3'b100: ALUControl_out = 4'b0011; // XOR[cite: 1] default: ALUControl_out = 4'b0010; endcase end default: ALUControl_out = 4'b0010; endcase end endmodule module maincontrol ( input [6:0] opcode, output reg RegWrite, output reg ALUSrc, output reg MemRead, output reg MemWrite, output reg MemtoReg, output reg Branch, output reg [1:0] ALUOp ); always @(*) begin // Default assignments to prevent latches RegWrite = 1'b0; ALUSrc = 1'b0; MemRead = 1'b0; MemWrite = 1'b0; MemtoReg = 1'b0; Branch = 1'b0; ALUOp = 2'b00; case (opcode) 7'b0110011: begin // R-type (ADD, SUB, SLL, SRL, AND, OR, XOR)[cite: 1] RegWrite = 1'b1; ALUOp = 2'b10; end 7'b0010011: begin // I-type Arithmetic (ADDI)[cite: 1] RegWrite = 1'b1; ALUSrc = 1'b1; ALUOp = 2'b00; end 7'b0000011: begin // I-type Load (LW, LH, LB)[cite: 1] RegWrite = 1'b1; ALUSrc = 1'b1; MemRead = 1'b1; MemtoReg = 1'b1; ALUOp = 2'b00; end 7'b0100011: begin // S-type Store (SW, SH, SB)[cite: 1] ALUSrc = 1'b1; MemWrite = 1'b1; ALUOp = 2'b00; end 7'b1100011: begin // B-type Branch (BEQ)[cite: 1] Branch = 1'b1; ALUOp = 2'b01; // Subtract for comparison end default: begin // Safe defaults end endcase end endmodule

Testbench:

`timescale 1ns / 1ps module tb_control_path(); // Testbench inputs reg [6:0] opcode; reg [2:0] funct3; reg funct7_5; // Outputs from Main Control wire RegWrite; wire ALUSrc; wire MemRead; wire MemWrite; wire MemtoReg; wire Branch; wire [1:0] ALUOp; // Output from ALU Control wire [3:0] ALUControl_out; // Instantiate Main Control Unit maincontrol uut_main ( .opcode(opcode), .RegWrite(RegWrite), .ALUSrc(ALUSrc), .MemRead(MemRead), .MemWrite(MemWrite), .MemtoReg(MemtoReg), .Branch(Branch), .ALUOp(ALUOp) ); // Instantiate ALU Control Unit Alucontrol uut_alu ( .ALUOp(ALUOp), .funct3(funct3), .funct7_5(funct7_5), .ALUControl_out(ALUControl_out) ); initial begin // Initialize Inputs opcode = 7'b0; funct3 = 3'b0; funct7_5 = 1'b0; #20; // 1. R-type: ADD[cite: 1] opcode = 7'b0110011; funct3 = 3'b000; funct7_5 = 1'b0; #20; // 2. R-type: SUB[cite: 1] opcode = 7'b0110011; funct3 = 3'b000; funct7_5 = 1'b1; #20; // 3. R-type: SLL[cite: 1] opcode = 7'b0110011; funct3 = 3'b001; funct7_5 = 1'b0; #20; // 4. R-type: SRL[cite: 1] opcode = 7'b0110011; funct3 = 3'b101; funct7_5 = 1'b0; #20; // 5. R-type: AND[cite: 1] opcode = 7'b0110011; funct3 = 3'b111; funct7_5 = 1'b0; #20; // 6. R-type: OR[cite: 1] opcode = 7'b0110011; funct3 = 3'b110; funct7_5 = 1'b0; #20; // 7. R-type: XOR[cite: 1] opcode = 7'b0110011; funct3 = 3'b100; funct7_5 = 1'b0; #20; // 8. I-type Arithmetic: ADDI[cite: 1] opcode = 7'b0010011; funct3 = 3'b000; funct7_5 = 1'b0; #20; // 9. I-type Load: LW / LH / LB[cite: 1] opcode = 7'b0000011; funct3 = 3'b010; funct7_5 = 1'b0; #20; // 10. S-type Store: SW / SH / SB[cite: 1] opcode = 7'b0100011; funct3 = 3'b010; funct7_5 = 1'b0; #20; // 11. B-type Branch: BEQ[cite: 1] opcode = 7'b1100011; funct3 = 3'b000; funct7_5 = 1'b0; #20; $finish; end endmodule

Waveforms:

## Task 3

## Design module  codes of top module:

module TopModule ( input wire clk, input wire btnC, input [15:0] sw, output reg [15:0] led ); wire [6:0] opcode = sw[6:0]; wire [2:0] funct3 = sw[9:7]; wire funct7_5 = sw[10]; wire w_RegWrite, w_ALUSrc, w_MemRead, w_MemWrite, w_MemtoReg, w_Branch; wire [1:0] w_ALUOp; wire [3:0] w_ALUControl_out; // Instantiate Main Control Unit maincontrol main_ctrl ( .opcode(opcode), .RegWrite(w_RegWrite), .ALUSrc(w_ALUSrc), .MemRead(w_MemRead), .MemWrite(w_MemWrite), .MemtoReg(w_MemtoReg), .Branch(w_Branch), .ALUOp(w_ALUOp) ); // Instantiate ALU Control Unit Alucontrol alu_ctrl ( .ALUOp(w_ALUOp), .funct3(funct3), .funct7_5(funct7_5), .ALUControl_out(w_ALUControl_out) );

## FSM:

localparam [1:0] IDLE = 2'b00, SAMPLE = 2'b01, UPDATE = 2'b10; reg [1:0] state = IDLE; reg btn_sync0 = 0, btn_sync1 = 0; always @(posedge clk) begin btn_sync0 <= btnC; btn_sync1 <= btn_sync0; end wire btn_pressed = btn_sync0 & ~btn_sync1; always @(posedge clk) begin case (state) IDLE: if (btn_pressed) state <= SAMPLE; SAMPLE: state <= UPDATE; UPDATE: begin led[0] <= w_RegWrite; led[1] <= w_ALUSrc; led[2] <= w_MemRead; led[3] <= w_MemWrite; led[4] <= w_MemtoReg; led[5] <= w_Branch; led[9:6] <= w_ALUControl_out; led[11:10] <= w_ALUOp; led[15:12] <= 4'b0000; state <= IDLE; end default: state <= IDLE; endcase end endmodule

Constraint file:
## Clock signal (100 MHz)

set_property PACKAGE_PIN W5 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk]

## Center Push Button (btnC)

set_property PACKAGE_PIN U18 [get_ports btnC]

set_property IOSTANDARD LVCMOS33 [get_ports btnC]

## Switches (sw[10:0])

set_property PACKAGE_PIN V17 [get_ports {sw[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[0]}]

set_property PACKAGE_PIN V16 [get_ports {sw[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[1]}]

set_property PACKAGE_PIN W16 [get_ports {sw[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[2]}]

set_property PACKAGE_PIN W17 [get_ports {sw[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[3]}]

set_property PACKAGE_PIN W15 [get_ports {sw[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[4]}]

set_property PACKAGE_PIN V15 [get_ports {sw[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[5]}]

set_property PACKAGE_PIN W14 [get_ports {sw[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[6]}]

set_property PACKAGE_PIN W13 [get_ports {sw[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[7]}]

set_property PACKAGE_PIN V2  [get_ports {sw[8]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[8]}]

set_property PACKAGE_PIN T3  [get_ports {sw[9]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[9]}]

set_property PACKAGE_PIN T2  [get_ports {sw[10]}]

set_property IOSTANDARD LVCMOS33 [get_ports {sw[10]}]

## LEDs (led[11:0])

set_property PACKAGE_PIN U16 [get_ports {led[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[0]}]

set_property PACKAGE_PIN E19 [get_ports {led[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[1]}]

set_property PACKAGE_PIN U19 [get_ports {led[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[2]}]

set_property PACKAGE_PIN V19 [get_ports {led[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[3]}]

set_property PACKAGE_PIN W18 [get_ports {led[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[4]}]

set_property PACKAGE_PIN U15 [get_ports {led[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[5]}]

set_property PACKAGE_PIN U14 [get_ports {led[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[6]}]

set_property PACKAGE_PIN V14 [get_ports {led[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[7]}]

set_property PACKAGE_PIN V13 [get_ports {led[8]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[8]}]

set_property PACKAGE_PIN V3  [get_ports {led[9]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[9]}]

set_property PACKAGE_PIN W3  [get_ports {led[10]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[10]}]

set_property PACKAGE_PIN U3  [get_ports {led[11]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[11]}]

set_property PACKAGE_PIN P3 [get_ports {led[12]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[12]}]

set_property PACKAGE_PIN N3 [get_ports {led[13]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[13]}]

set_property PACKAGE_PIN P1 [get_ports {led[14]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[14]}]

set_property PACKAGE_PIN L1 [get_ports {led[15]}]

set_property IOSTANDARD LVCMOS33 [get_ports {led[15]}]

RTL Schematic:

# Lab 9: Design and FPGA Implementation of the Control Path for a Single-Cycle RISC-V Processor

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | LR11 Design | AR7 |
| --- | --- | --- | --- | --- |
| Task 1 | - | - | /20 | /20 |
| Task 2 | /20 | /15 |  |  |
| Task 3 | /10 | /15 |  |  |
| Total Points | /80 |  |  | /20 |
| CLO Mapped | CLO 1 |  |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |